CO- 2: AT – 2: Simulation Task

Name;C.ramakrishnareddy

Q1) 31. Simulation of propagation loss under 700 MHz, 3.5 GHz, 28 GHz, 38 GHz, and 60 GHz

Objective1

Matlab code;

clc;

clear;

close all;

% Frequencies (Hz)

freq = [700e6 3.5e9 28e9 38e9 60e9];

% Distance (m)

d = 1:1:1000;

figure;

hold on;

for i = 1:length(freq)

PL = 20*log10(d) + 20*log10(freq(i)) - 147.55; % FSPL

plot(d, PL, 'LineWidth',2);

end

grid on;

xlabel('Distance (m)');

ylabel('Path Loss (dB)');

title('Propagation Loss at Different Frequencies');

legend('700 MHz','3.5 GHz','28 GHz','38 GHz','60 GHz');

output;

Q2) Simulation of mmWave path loss under distance, frequency, atmospheric absorption,

blockage, and antenna gain

Objective1

Matlab code

clc;

clear;

close all;

d = 1:1:500;

f = 28e9;

atm_loss = 0.015;      % dB/m

blockage = 20;         % dB

Gt = 20;               % Transmitter gain

Gr = 20;               % Receiver gain

FSPL = 20*log10(d)+20*log10(f)-147.55;

TotalLoss = FSPL + atm_loss*d + blockage - Gt - Gr;

plot(d,TotalLoss,'r','LineWidth',2);

grid on;

xlabel('Distance (m)');

ylabel('Path Loss (dB)');

title('mmWave Path Loss');

Output;

Q3) Simulation of channel modeling for mmWave MIMO under LOS, NLOS, fading, delay

spread, and SNR

Objective1;

Matylab code;

clc;

clear;

close all;

Nt = 4;

Nr = 4;

SNR = 0:2:30;

Capacity_LOS = zeros(size(SNR));

Capacity_NLOS = zeros(size(SNR));

for k=1:length(SNR)

% LOS Channel

H1 = ones(Nr,Nt);

% NLOS Channel

H2 = (randn(Nr,Nt)+1i*randn(Nr,Nt))/sqrt(2);

rho = 10^(SNR(k)/10);

Capacity_LOS(k)=log2(det(eye(Nr)+rho/Nt*(H1*H1')));

Capacity_NLOS(k)=real(log2(det(eye(Nr)+rho/Nt*(H2*H2'))));

end

plot(SNR,Capacity_LOS,'b-o','LineWidth',2);

hold on;

plot(SNR,Capacity_NLOS,'r-*','LineWidth',2);

grid on;

xlabel('SNR (dB)');

ylabel('Capacity (bits/s/Hz)');

title('mmWave MIMO Channel');

legend('LOS','NLOS');

Output;

Q4) Simulation of Interference Management under User Density, Reuse Factor, SNR, SINR, and Throughput

Objective1;

Matlab code;

clc;

clear;

close all;

users = 10:10:200;

reuse = 3;

SNR = 20;

SINR = zeros(size(users));

Throughput = zeros(size(users));

for i=1:length(users)

interference = users(i)/reuse;

SINR(i)=SNR/(1+interference);

Throughput(i)=log2(1+SINR(i));

end

subplot(2,1,1)

plot(users,SINR,'LineWidth',2)

grid on

xlabel('Number of Users')

ylabel('SINR')

subplot(2,1,2)

plot(users,Throughput,'r','LineWidth',2)

grid on

xlabel('Number of Users')

ylabel('Throughput (bps/Hz)')

output;

Q5) Simulation of Fading Channel Effects under Rayleigh, Rician, Nakagami, BER, and Outage Probability

Objective;

Matlab code;

clc;

clear;

close all;

SNRdB = 0:2:30;

snr = 10.^(SNRdB/10);

%% Rayleigh BER

BER_Rayleigh = 0.5*(1-sqrt(snr./(1+snr)));

%% Rician BER (Approximation)

K = 5;

BER_Rician = BER_Rayleigh./(K+1);

%% Nakagami BER (m=2 Approximation)

m = 2;

BER_Nakagami = (0.5*(1-sqrt(snr./(m+snr))));

%% Outage Probability

threshold = 5;

Outage = 1-exp(-threshold./snr);

subplot(2,1,1)

semilogy(SNRdB,BER_Rayleigh,'b','LineWidth',2)

hold on

semilogy(SNRdB,BER_Rician,'r','LineWidth',2)

semilogy(SNRdB,BER_Nakagami,'k','LineWidth',2)

grid on

xlabel('SNR (dB)')

ylabel('BER')

legend('Rayleigh','Rician','Nakagami')

subplot(2,1,2)

plot(SNRdB,Outage,'m','LineWidth',2)

grid on

xlabel('SNR (dB)')

ylabel('Outage Probability')

title('Outage Probability')

output;